EOG-Based Assistive Navigation System

Project Work Plan  |  Revised Scope  |  2025

Team: Jatinderpal Singh | Sambhav Manhas

# 1. Project Overview

An EOG (Electrooculography) based assistive navigation system that allows users to control a computer entirely through eye signals — blinks and winks. The system replaces traditional motor imagery EEG with reliable, affordable eye-movement detection and integrates Claude AI to intelligently interpret signals and execute OS-level navigation actions.

This is a pivot from the original motor imagery BCI concept. The new system is fully achievable with existing hardware and produces a working, demonstrable product.

# 2. What Is New in This Version

## 2.1 Navigation via EOG signals only

The system no longer attempts motor imagery (EEG). All control is through eye-based EOG signals which are 10–50x stronger and far more reliable with consumer-grade hardware.

## 2.2 Claude AI as the decision engine

Instead of hard-coded if/else logic, each detected signal is sent to Claude API with the current UI state. Claude decides what action to take based on context — making the system adaptive and intelligent rather than rigid.

## 2.3 Scanning interface for navigation

A custom scanning UI cycles through navigation options (folder, browser, camera, notepad, image viewer, etc.) automatically. The user blinks to select the highlighted item — no direction signal required.

## 2.4 Full OS navigation capability

The system can open folders, images, camera, browser, and other applications using Python subprocess calls — fulfilling the complete navigation requirement specified by the project supervisor.

# 3. Signal Mapping

| Signal | EOG Detection | Navigation Action | Reliability |
| --- | --- | --- | --- |
| Single blink | Large spike on Fp1, both eyes | Select highlighted item | High |
| Double blink | Two spikes within 600ms | Go back / cancel | High |
| Wink left | Asymmetric deflection, left side | Move highlight to previous | Medium |
| Wink right | Asymmetric deflection, right side | Move highlight to next | Medium |

# 4. New Data Collection Plan

## 4.1 Electrode placement (new)

- IN+ on Fp1 (left forehead, above left eyebrow)

- IN− on left earlobe (referential setup, not bipolar)

- GND on right earlobe

This referential Fp1 setup captures clean unipolar EOG signals and clearly distinguishes left wink, right wink, and bilateral blink.

## 4.2 Data collection protocol

- Run test_blink_wink.py — subject 002 (new session, separate from old data)

- 75 trials total: 25 blinks, 25 wink-left, 25 wink-right

- Pygame window guides the user with eye icons on screen

- Rest 5 seconds between each block to reduce eye fatigue

- Collect minimum 2 sessions on different days for robustness

## 4.3 Data quality checks

- Inspect raw signal in real time — blink peaks should exceed 3x baseline amplitude

- Reject trials where electrode contact was poor (flat or noisy signal)

- Ensure DC offset is accounted for before peak detection threshold is set

## 4.4 Model retraining

- Train new eog_1ch_model.pkl using train_eog_1ch.py on fresh Fp1 data

- Target: ≥80% cross-validation accuracy before integration

- Test live with mouse_control_eog.py in simulate mode first, then real hardware

# 5. Phase-wise Work Plan

| Phase | Task | Key Deliverable | Status |
| --- | --- | --- | --- |
| Phase 1 | New electrode setup + data collection (Fp1 referential) | 75+ labelled EOG trials saved as subject_002 | Pending |
| Phase 2 | Train new 1-channel EOG classifier | eog_1ch_model.pkl with ≥80% accuracy | Pending |
| Phase 3 | Build scanning navigation UI | Python UI that cycles through 6 OS actions | In progress |
| Phase 4 | Integrate Claude API decision layer | Claude interprets signal + context, returns action | In progress |
| Phase 5 | Connect EOG model output to navigation + OS execution | Full pipeline: blink → Claude → open folder/browser/camera | Pending |
| Phase 6 | Demo preparation + fallback keyboard mode | Recorded demo video + keyboard backup for exam day | Pending |

# 6. Technology Stack

| Layer | Component | Detail |
| --- | --- | --- |
| Hardware | BioAmp EXG Pill + Arduino R4 Minima | Single pill, Fp1 referential, COM7, 500Hz |
| Signal processing | Python + scipy + numpy | Bandpass filter, peak detection, threshold classifier |
| ML model | SVM (eog_1ch_model.pkl) | 3-class: blink / wink_left / wink_right |
| AI decision layer | Claude API (claude-sonnet-4-5) | Signal + UI state → action decision in natural language |
| Navigation UI | Python (tkinter or pygame scanning UI) | Auto-cycling highlight, 6 OS actions |
| OS execution | Python subprocess | Opens folder, browser, camera, notepad, image viewer |

# 7. Demo Script for Examination

The demo will show a complete end-to-end navigation session without any manual mouse or keyboard input (except for the keyboard fallback if hardware fails).

## Demo sequence

- Start scanning UI — 6 options highlighted one by one automatically

- Wink right twice → highlight moves to Open Browser

- Blink once → browser opens (Chrome/Edge)

- Wink left → highlight moves back to Open Folder

- Blink once → File Explorer opens

- Double blink → returns to main menu

- Wink right → highlight moves to Open Camera

- Blink once → Windows Camera app launches

## Fallback plan

- Keyboard keys (B, W, X, D) simulate blink/wink signals for demo if hardware glitches

- Pre-recorded demo video as backup if live hardware fails entirely

# 8. Key Risks and Mitigations

| Risk | Mitigation | Fallback |
| --- | --- | --- |
| Poor electrode contact during demo | Wet scalp, press firmly, use tape to hold electrode | Keyboard simulation mode |
| Wink detection unreliable | Switch to auto-scan mode — only blink needed | Auto-scan removes need for wink signals |
| Claude API call fails or is slow | Add local fallback decision logic in Python | Direct signal-to-action mapping without API |
| Insufficient training data | Collect 2+ sessions, augment with noise injection | Use pre-existing eog_3class_model.pkl |

This document is a living plan — update phase statuses as work progresses. All previous motor imagery data and EEG-based models are deprecated and not part of this revised scope.