# NavTools: EOG-Based Assistive Navigation

# Project Presentation & Demonstration Guide

This document outlines the final working implementation of the EOG-based assistive navigation system, designed for hands-free computer interaction.

## 1. Executive Summary

NavTools is a specialized assistive interface that allows users with limited motor function to control a computer using only eye movements. By capturing Electrooculography (EOG) signals from the forehead (Fp1 position), the system classifies blinks based on their temporal dynamics (duration and frequency) to navigate a custom application dashboard.

## 2. System Architecture (How it Works)

The system is split into three modular layers:

Hardware Layer:

BioAmp EXG Pill: A high-precision analog front-end that captures micro-volt level electrical changes from the eyes.

Arduino R4 Minima: Digitizes the signal and streams it to the PC at 500Hz.

Processing Layer (Python Engine):

Adaptive Baseline Tracking: Automatically adjusts to signal drift and electrode impedance changes.

Temporal Classification: Uses physics-based duration analysis instead of unreliable AI models.

HTTP Control Link: Sends commands directly to the UI over a local network socket (Port 7891).

Application Layer (NavTools UI):

Electron-based Interface: A modern, high-contrast dashboard optimized for assistive use.

Direct IPC Integration: Receives Python commands instantly, even if the window is not in focus.

## 3. Demonstration Script (Live Walkthrough)

Step 1: Preparation

Ensure electrodes are placed at Fp1 (above left eyebrow), Fp2 (above right eyebrow - optional/reference), and Earlobe (Ground).

Launch the NavTools UI (`npm start`).

Launch the EOG Controller (`python -m src.navtools_eog_control --port COM7`).

Step 2: Calibration (The "Wow" Factor)

Show the director the "Baseline Tracking" in the terminal.

Explain: *"The system takes 3 seconds to learn your unique eye physiology and current noise floor."*

Step 3: Navigation Demo

Single Quick Blink: Move the selection right through the apps.

*Explain*: *"A short blink (80-500ms) triggers a 'Next' command."*

Long Hold Blink: Move the selection back to the left.

*Explain*: *"Holding the eyes shut for >600ms reverses direction. This is more reliable than trying to detect left/right eye look with a single electrode."*

Double Blink: Select an app (e.g., Calculator or Browser).

*Explain*: *"Two quick blinks in rapid succession trigger the 'Select' action. Our 'Refractory Window' logic prevents accidental triggers from natural eye fatigue."*

## 4. Anticipated Q&A (Director Level)

Q1: Why use duration (Short/Long) instead of just Left/Right eye winks?

Answer: *"Most single-channel hardware (Fp1) produces nearly identical signal polarities for both left and right winks. By using duration, we create a robust, hardware-agnostic control scheme that doesn't require complex multi-channel setups or expensive medical-grade sensors."*

Q2: How does the system handle 'Noise' (e.g., the user just blinking naturally)?

Answer: *"We implemented a 300ms Refractory Window and an 80ms Minimum Artifact Filter. Natural micro-blinks are ignored, and the 'biphasic recovery' of the signal is filtered out so it isn't miscounted as a double-click."*

Q3: Can this control other Windows applications?

Answer: *"Yes. While we developed a custom UI for maximum accessibility, the Python engine can easily be mapped to generic keyboard emulation (Arrow Keys/Enter) to control any standard software."*

Q4: What is the benefit of the HTTP Control over standard Keyboard simulation?

Answer: *"Security and Reliability. Keyboard simulation requires the window to be 'In Focus'. Our HTTP/IPC architecture allows NavTools to receive commands even if it's in the background, making it a true background service."*

## 5. Technical Specifications

Sampling Rate: 500 Hz

Processing Latency: < 20ms

Communication: Local HTTP (127.0.0.1:7891)

Gesture Thresholds:

Short: 80ms - 599ms

Long: 600ms+

Double: < 800ms gap